Briefing Document: 6-Color Servo Stack Type Flexo Printing Machine

Commercial Terms and Pricing

The quotation, issued on June 22, 2026, details the investment requirements and logistical timelines for the Honor Series equipment.

Technical Specifications and Performance

6-Color Servo Stack Type Flexo Printing Machine is engineered for high-precision resin plate printing with a focus on stability and speed.

General Machine Parameters

- Material Compatibility:  Paper, PP lamination bag etc.

- Printing Dimensions:  800mm material feeding width; 760mm maximum printing width.

- Print Repeat Range:  300 mm – 1000 mm.

- Operating Speed:

- Maximum Machine Speed: 230 m/min.

- Maximum Printing Speed: 150–200 m/min (variable based on material and pattern).

- Precision:  Register precision of ±0.2 mm.

- Physical Footprint:  10m x 2.7m x 4.3m.

- Power Requirements:  Total machine power of 130kw; utilizes electricity heating.

Core Component Manufacturers

The machine integrates components from several recognized industrial suppliers to ensure reliability:

- Servo Motors:

- Electrical Components (Relays/Switches):  Schneider.

- Pneumatics:  Airtac (electromagnetic valves) and Fujikura Japan (low friction cylinders).

- Anilox Rollers:  Taiwan made (Ceramic, 80-1000LPI).

- Monitoring:  Taiwan (Video inspector).

System Architecture and Key Features

1. Unwinding and Rewinding Units

- Unwind:  Employs active shaftless unwinding (3-inch and 6-inch) clamped by a two-head cylinder system. It features a Φ1200mm maximum diameter and an air brake for tension control.

- Web Guide:  A double-roller EPC (Electronic Power Control) automatic correction system is installed before printing to manage lateral translation.

- Rewind:  Utilizes friction rewind with a wing arm for smoother material collection, supporting a Φ1200mm maximum diameter.

2. Printing Unit Capabilities

- Versatility:  Supports multiple printing modes including 6+0 (single-side), 3+3, 4+2, and 3+3 (double-sided).

- Servo Integration:  Each printing unit utilizes four servo motors to control the feed and movement of the anilox and printing rollers. This allows for precise pressure calculation and rapid pre-pressure setup.

- Doctor Blade System:  Features the latest generation chamber doctor blade with venting and pressure-controlled cylinder clamping to reduce ink splatter and air bubbles.

- Registration:  A servo motor control system enables 360-degree color registration, reducing manual intervention.

- Mechanical Rigidity:  100mm frame thickness and guide rails with intermediate pre-pressure eliminate gaps and increase movement smoothness.

3. Drying and Monitoring Systems

- Drying Unit:  A recirculating air system with frequency-controlled dual fans. It includes a color-reversing drying oven at the bottom and achieves heat recovery to save energy.

- Static Image Monitoring:  The Arise video inspector provides 3-30x magnification. The camera features a patrol speed of 1.0 m/min and can be set for fixed-point or automatic reciprocating motion.

- HMI:  A PLC-driven touchscreen interface handles data modification, historical logging, and alarm notifications.

Optional Auxiliary Equipment

The following items are available based on customer requirements:

- Plate Mounting Machine:  Features high-strength wallboard construction and 650-line HD cameras (70x amplification) for precision mounting.

- Ultrasonic Cleaning Machine:  Includes 36 vibrators and a temperature range of 30℃-100℃ for maintaining anilox rollers.

- Ink Viscosity Control:  An automatic system using a vibration sensor to maintain solvent/water balance.

- AI Online Detector:  Real-time defect detection for ink spots, overprint errors, and barcode/QR code integrity.

Installation and Support

Wenzhou Lisheng provides technical support for the commissioning of the production line:

- Personnel:  One engineer provided for 12 days to manage installation, testing, and worker training.

- Buyer Responsibilities:  The buyer must cover the engineer's salary ($120/day), visa, airfare, lodging, and local transportation. Additionally, the buyer must provide necessary labor, external wiring, and air compressors.

- Manufacturing Standards:  The company utilizes Mazak and gantry machining centers to ensure the precision of individual machine components.